# Fusion 360

tutorial

# shortcuts

Line – L

Circle – c

# Create rectangle box

## Create project

## Change active unit

## New component

Assemble->new component

Name : lego

## Create 2 point rectangle

Create new Sketch(left side + icon) -> select a plane -> select two point rectangle (r)

- Click the origin

- Drag

- 15.8 in x and press tab

- 31.8 in y

- Extrude(e) – 9.8

## Create circle

Create new sketch -> select top of rectangle -> create circle(c)

Press (d) for dimension -> select center of circle and top edge of rectangle -> click and drag out the dimension

- Right click -> repeat sketch dimension

- Set the same dimension in vertical

- Extrude (e) -> select circle -> 1.7

## Create rectangle pattern

- On solid tab -> pattern -> create rectangle pattern

- Select regular pattern

- Select features

- Objects -> select extrude in bottom

- Select direction – 24 x 4

- Select other direction

- 8 x 2

## shell

- Modify -> shell

- Faces/body select bottom face

- Inside thickness 1.49

## Create sketch inside face

Right click inner face

- Create sketch

- Press (l) for line -> construction

- Select 1st row second circle center point

- Select 2nd row 1st circle center point

- Press (c) for center circle

- Off construction in dialog box

- Select mid point of line

- Select the tangent of 2nd circle

- Press O for offset

- Select circle -> set diameter 1

- Flip the circle inside

- Press e for extrude

- Select circle

- Extrude 8.1

Select pattern

- Create ->pattern -> rectangular pattern

- Distance 16 x 3

## Remove sharpen edge – fillet(f)

Modify -> fillet (f)

- Select edges

- 0.2

# Bottle

tutorial

- Create new component

  - On solid plane

  - Create

  - New component

  - Name: beer bottle

- Attatch bottle image

  - Download image

  - Insert -> canvas -> open image -> select xy plane -> rescale -> ok

- Calibrate image

  - Browser -> beer bottle -> beer bottle -> canvas -> select image

  - Right click image ->calibrate

  - In 3d view-> select front

  - Select the bottom right point, top right point

  - Set dimension 240

- Create line

  - Press L

  - Select top mid point

  - Select bottom mid point, continue

  - Set dimension 30 select bottom right point, continue

  - Set 134, select top right corner

- Sketch point

  - Create -> point

  - Select other sides

- Spline

  - Create -> spline-> fit point spline

- Revolve

  - Solid

  - Create

  - Revolve

  - Select pivot

  - Select axis

- Fillet

  - Press f

  - Select bottom circle – 5

  - Select top circle – 1

- Make top as hollow

  - Double click sketch in bottom left corner(menu)

  - Modify -> shell ->

  - Browse -> Bodies -> Body

  - In shell thickness 3

  - Inspenct -> section analysis

  - Browser -> origin -> xz

  - Select top face

  - Press c

  - Click the center of bottle top

  - Drag 14 mm

  - Select circle

  - Extrude e

  - Operation cut

  - Extent – two object selection or scale the length up -> ok

- Fillet the inner edge to 2

- Select -> browse -> analysis

- Glass appearance

  - Deselect browse -> canvases

  - Select body

  - Right click -> appearance

  - Drag and drop glass appearance

# Paper clip

- Press l

- Select xy

- Click origin

- Drag top 20 mm

- Press s for shortcut search

- Search fillet -> select 2d fillet

- Select top 2 corners

- Mark 3.75

- Right click -> repeat fillet -> select bottom 2 corners 3.25

- Select last 2 corner 2.75

- Select finish sketch in top right menu

- Press c

- Select xz

- Select origin -> drag -> 1mm

- Select home icon

- Sweep

  - Create-> sweep

  - Profile -> circle

  - Path -> line

# Whiskey bottle

- Assembly -> new component -> name: whiskey bottle

- Press c -> xy plane

- Press r -> center rectangle -> w: 76, h:63

- Press s

  - Search fillet

  - Select sketch fillet

  - Select all edges : 7mm

  - Finish sketch

- Construct -> offset

  - Select xy plane

  - Distance: 114

- Press r

  - Select 2nd plane

  - Center rectangle

  - W:95, h:76

- Search s

  - Select 4 corners

  - 12

  - Finish sketch

- Construct -> offset plane -> d:38

- Press c

  - Select 3rd plane

  - Radius 40mm

  - Finish sketch

- Construct -> offset plane

  - Select 3rd plane

  - D:89mm

- Press c

  - Select 4th plane

  - R:30

- Create

  - Loft

  - Select bottom to top

  - Cancel

- Select first sketch in timeline (bottom menu)

  - Select line from toolbar(l)

  - Select construction in sketch pallet

  - Select mid point in right side of rec

  - Select mid point of left side of rec

  - Finish sketch

- Repeat step 13 for 2nd sketch

- Select 3rd sketch

  - Press l

  - Pres x

  - Select mid point of circle

  - Select left side

  - Finish sketch

- Repeat step 15 for sketch 4

- S

  - S: Split

  - Fit point spline

  - Press xz plane

  - Select 1st 3 points

  - enter

  - Adjust radius

- Press l

  - Select 3rd and 4th point

- Select end of 3rd point

  - Make it as mid point of last line

- Press l

  - Click bottom of the line

- Create -> mirror

  - Select two sides

  - Mirror line is the vertical center line

- Create

  - Loft

  - Press home icon to isotropic view

  - Select bottom to top

  - Select + icon in guide rule

  - Select line 1 and press + icon select line 2

- F

  - Select bottom circle

  - 5mm

- C

  - Select top surface

  - D:27

- E

  - Select top circle

  - 12mm

- Modify

  - Shell

  - Select top circle

  - 3mm

- Thread

  - Create

  - Thread

  - Check modeled : true

  - Uncheck full length: drag down one thread: 10mm

  - Ok

- Select body

  - Right click appearance

  - Drag glass bronze

- In bottom menu

  - Display

  - Environment

  - Dark theme